# Dynamic Gating Analysis Tool

# Summary

A Python-based GUI application for flow cytometry data analysis that performs 2D dynamic gating with CDF shape matching. The program optimizes gate position on one fluorescence channel (X-axis) while keeping the gate fixed on another channel (Y-axis) across time-series measurements. The optimization finds the gate position that minimizes the difference between CDF shapes (centered, position-independent comparison) of the gated populations.

# Key Features

2D cytogram visualization with interactive rectangular gate selection

CDF shape-based optimization - finds optimal X-gate position by minimizing L2 norm of centered CDF differences

Fixed Y-gate across all time points

Selectable reference time point (not necessarily the last one)

Optional fraction matching - penalizes deviations in the percentage of events in the gate

Kinetics fitting with A + B·(1 - exp(-t/τ)) model for both Mean X and Gate Position

Comprehensive visualization: cytograms, histograms, CDF comparisons, search curves, kinetics plots

Results export to CSV files

# User Guide

## 1. Load Data Files

Click "Add Time Point Files" button

Select your FCS/LMD files (multiple selection allowed)

In the dialog, enter the time (in minutes) for each file

Files are sorted by time automatically

## 2. Select Channels

Y-axis channel (FIXED gate): Select the channel for the fixed gate (e.g., FSC-A, SSC-A)

X-axis channel (OPTIMIZED position): Select the channel whose gate position will be optimized (e.g., FL1-A)

Log transform options are not applied - raw data is used directly

## 3. Set Reference Gate

Select the reference time point from the dropdown list

Click "Set Reference Gate (2D cytogram)"

In the new window, click and drag with mouse to draw a rectangular gate

Use the toolbar (zoom/pan) if needed for precise selection

The statistics panel shows gate boundaries, event count, fraction, mean, and sigma

Click "Save Gate" to confirm

## 4. Configure Search Parameters

X search range (± units): Range to search for optimal X-gate position (default: 100000)

Number of search steps: More steps = finer search but slower (default: 200)

Step size is automatically calculated and displayed

## 5. Optional Settings

Match gate fraction: When checked, the optimization penalizes deviations in the percentage of events in the gate compared to the reference

Fraction penalty weight: Adjustable weight for this penalty (default: 10.0)

Fit Mean X: Fits A + B·(1 - exp(-t/τ)) to mean X values vs time

Fit Gate Position: Fits A + B·(1 - exp(-t/τ)) to gate center position vs time

## 6. Run Analysis

Click "Run Dynamic Gating Analysis" to start the optimization.

The program will:

Apply the fixed Y-gate to all time points

For each non-reference time point, search for the X-gate position that minimizes CDF shape difference

Calculate mean and sigma for each optimized gate

Fit kinetics models (if enabled)

Display results in the plots and tables

## 7. View Results

The results are displayed in 7 tabs:

| Tab | Description |
| --- | --- |
| 2D Cytograms | Scatter plots with gate rectangles (green=reference, red=optimized) |
| Histograms (X) | X-channel distributions after Y-gating with gate boundaries |
| CDF Shape Comparison | Centered CDFs (shape comparison) and original CDFs |
| Gate Search | Shape score and KS distance vs X-shift curves |
| Kinetics (Mean X) | Mean X values vs time with exponential fit |
| Kinetics (Gate Position) | Gate center position vs time with exponential fit |
| Results Table | Summary of all parameters and results |

## 8. Export Results

Click "Export Results" to save CSV files:

gate_results.csv - Gate positions, statistics for all time points

mean_kinetics_fit.csv - Fitted parameters for Mean X kinetics (if enabled)

position_kinetics_fit.csv - Fitted parameters for Gate Position kinetics (if enabled)

search_t*.csv - Search curves for each time point

# Algorithm Details

Reference gate is defined on the selected time point (X-range and Y-range)

For each other time point: Y-gate is applied as-is (same absolute values), X-gate position is searched by trying many small shifts

For each shift, the CDF of gated X-values is computed and centered (shifted to start at 0)

Shape score = L2 norm of difference between reference and target centered CDFs

Optional fraction penalty = weight × |target_fraction - reference_fraction|

Optimal shift = position with minimum combined score

Gate width is preserved throughout the optimization

# Requirements

Python 3.7+

numpy, scipy, pandas, matplotlib, tkinter

Optional: FlowCytometryTools, fcsparser (for FCS file reading)

# Tips

For linear-scale data (e.g., FSC values), use a larger search range (e.g., 100000)

For log-scale data, convert before loading or use smaller search range

More search steps give smoother optimization curves but take longer

The reference time point can be any point, not necessarily the last one

Gate fraction matching is useful when you expect similar percentages of gated events across time points